Microchip dsPIC Based Drone ESC

Project Garuda

Clean-Room ESC Firmware Implementation

for Microchip dsPIC33AK Digital Signal Controller

Comprehensive Technical Specification

Clean-Room Implementation Matching Industry-Standard ESC Feature Sets

Version 1.3

January 2026

REVISION HISTORY

| Version | Changes |
| --- | --- |
| 1.0 | Initial specification |
| 1.1 | Added: Clean-room appendix, bidir checksum, state machine, timing windows, EEPROM layout, bootloader, verification plan |
| 1.2 | Added: Section 7 Configurator Protocol (GSP, MSP passthrough, bootloader commands, dead time calibration). Renamed to Project Garuda. |
| 1.3 | Added: Section 3.3 Advanced Algorithms (flux linkage, ternary hysteresis, adaptive lead angle, mixed decay, CLC state machine) |

Prepared for: Microchip Technology Inc. - Applications Engineering, South Asia

# Table of Contents

# 1. Executive Summary

This document provides a comprehensive clean-room implementation specification for Project Garuda - a high-performance ESC (Electronic Speed Controller) firmware developed on Microchip's dsPIC33AK Digital Signal Controller family. The specification achieves feature parity with industry-standard ESC firmware while maintaining complete IP independence through rigorous clean-room development practices.

## 1.1 Project Objectives

- Develop original, proprietary ESC firmware with complete IP ownership

- Achieve full protocol compatibility with DShot, bidirectional DShot, and EDT telemetry

- Leverage dsPIC33AK's 200MHz CPU, hardware FPU, and 40 MSPS ADC

- Support functional safety requirements (IEC 61508, ISO 26262)

- Enable Make-in-India drone component ecosystem development

## 1.2 Key Differentiators

| Feature | ARM-based ESC | dsPIC33AK ESC | Advantage |
| --- | --- | --- | --- |
| CPU Speed | 48-72 MHz | 200 MHz | 3-4x faster |
| Floating Point | Software emulation | Hardware DP-FPU | 100x faster FP |
| ADC Speed | ~1 MSPS | 40 MSPS | 40x faster |
| Comparator Delay | ~50 ns | 5 ns | 10x faster |
| PWM Resolution | ~20 ns | 78 ps (HRPWM) | 256x finer |

# 2. Communication Protocol Specifications

## 2.1 DShot Protocol Overview

DShot is a digital protocol for flight controller to ESC communication. Each frame consists of 16 bits transmitted using pulse-width encoding.

### 2.1.1 DShot Timing Specifications

| Protocol | Bit Rate | Bit Period | T0H | T1H | Frame Time |
| --- | --- | --- | --- | --- | --- |
| DShot150 | 150 kbit/s | 6.67 µs | 2.50 µs | 5.00 µs | 106.7 µs |
| DShot300 | 300 kbit/s | 3.33 µs | 1.25 µs | 2.50 µs | 53.3 µs |
| DShot600 | 600 kbit/s | 1.67 µs | 625 ns | 1250 ns | 26.7 µs |
| DShot1200 | 1200 kbit/s | 0.83 µs | 313 ns | 625 ns | 13.3 µs |

### 2.1.2 Frame Structure and CRC

16-bit frame: [Throttle/Command: bits 15-5] [Telemetry: bit 4] [CRC: bits 3-0]

CRC Calculation: crc = (value ^ (value >> 4) ^ (value >> 8)) & 0x0F

Where 'value' is the 12-bit data (throttle + telemetry bit, i.e., frame >> 4).

### 2.1.3 DShot Command Table

| Value | Command | Repeats | Notes |
| --- | --- | --- | --- |
| 0 | DISARM | 1 | Motor disarmed, safe state |
| 1-5 | BEEP_1 to BEEP_5 | 1 | Different beep frequencies |
| 6 | ESC_INFO | 1 | Request ESC info (KISS protocol) |
| 7 | SPIN_DIRECTION_NORMAL | 6x | Set normal rotation |
| 8 | SPIN_DIRECTION_REVERSED | 6x | Set reversed rotation |
| 9/10 | 3D_MODE_OFF/ON | 6x | Enable/disable bidirectional mode |
| 12 | SAVE_SETTINGS | 6x | Save to EEPROM |
| 13/14 | EDT_ENABLE/DISABLE | 6x | Enable/disable Extended Telemetry |
| 48-2047 | THROTTLE | 1 | Active throttle (2000 steps) |

## 2.2 Bidirectional DShot (RPM Telemetry)

Bidirectional DShot enables eRPM telemetry over the same signal wire. The protocol uses inverted polarity (idle HIGH) to indicate bidirectional mode. Note: This differs from classic ESC telemetry which uses a separate UART wire.

### 2.2.1 Bidirectional Mode Detection

- Standard DShot: Idle state is LOW, bit encoding is non-inverted

- Bidirectional DShot: Idle state is HIGH, bit encoding is inverted

- ESC SHALL detect mode by observing idle polarity before first valid frame

- Mode detection SHALL complete within 10ms of signal presence

### 2.2.2 Checksum Handling (Critical Compatibility Detail)

FC → ESC (Receive) - Inverted DShot:

- CRC is calculated on the uninverted data value

- CRC itself is transmitted inverted (like all bits in bidir mode)

- ESC SHALL invert received bits before CRC validation

ESC → FC (Transmit) - GCR Telemetry:

- CRC is calculated on the 12-bit telemetry data (uninverted)

- CRC is transmitted uninverted within the GCR-encoded frame

### 2.2.3 Bidirectional DShot Test Vectors

Implementers SHALL verify CRC computation against these test vectors:

| Throttle | Telemetry Bit | CRC | Full Frame (hex) |
| --- | --- | --- | --- |
| 0 (disarm) | 0 | 0x0 | 0x0000 |
| 48 (min throttle) | 0 | 0x3 | 0x0603 |
| 1047 (mid) | 0 | 0x7 | 0x82F7 |
| 2047 (max) | 0 | 0xF | 0xFFEF |
| 1047 (mid) | 1 (request) | 0x6 | 0x82E6 |

### 2.2.4 eRPM Telemetry Frame

16-bit response: [Exponent: bits 15-13] [Mantissa: bits 12-4] [CRC: bits 3-0]

eRPM = Mantissa << Exponent; Actual RPM = eRPM / (Motor_Poles / 2)

### 2.2.5 Timing Requirements

| Parameter | Value | Tolerance |
| --- | --- | --- |
| FC frame end to ESC response start | 30 µs | ±5 µs |
| GCR response duration (21 bits) | 28 µs (DShot600) | Per bitrate |
| Total cycle (DShot600 bidir) | ~115-170 µs | ~5.9-8.7 kHz max |

## 2.3 Extended DShot Telemetry (EDT)

EDT extends bidirectional DShot by encoding additional telemetry within frames where eRPM normalization leaves unused encoding space.

### 2.3.1 eRPM Normalization and EDT Detection

- If (exponent == 0) OR (mantissa bit 8 == 1): Frame is eRPM

- If (exponent != 0) AND (mantissa bit 8 == 0): Frame is EDT

### 2.3.2 EDT Frame Types

| Prefix (4 bits) | Frame Type | Value Encoding |
| --- | --- | --- |
| 0010 (001 0) | Temperature | 0-255: Direct value in °C |
| 0100 (010 0) | Voltage | 0-255: Value × 0.25V (range 0-63.75V) |
| 0110 (011 0) | Current | 0-255: Direct value in Amperes |
| 1000 (100 0) | Debug 1 | Firmware-defined debug value |
| 1010 (101 0) | Debug 2 | Firmware-defined debug value |
| 1100 (110 0) | Stress Level | 0-255: Commutation difficulty (0=perfect, 255=failing) |
| 1110 (111 0) | Status | Bit7=alert, Bit6=warning, Bit5=error, Bit3:0=max stress |

### 2.3.3 EDT Scheduling Policy (Default, Configurable)

| Frame Type | Interval | Priority / Notes |
| --- | --- | --- |
| eRPM | Default (filler) | Sent when no EDT frame pending |
| Status | 1000 ms OR on event | Highest priority; immediate on error/warning/alert |
| Temperature | 1000 ms | Offset +333 ms from status |
| Voltage | 1000 ms | Offset +666 ms from status |
| Current | 1000 ms (if available) | Offset +833 ms from status |

Configuration Knobs:

- Base interval (default 1000, range 100-5000)EDT_PERIOD_MS:

- Maximum EDT frames per second (default 10, range 1-50)EDT_RATE_CAP:

- Send status on error/warning immediately (default ON)EDT_EVENTS_IMMEDIATE:

# 3. Motor Control Algorithm

## 3.1 Firmware State Machine

The ESC firmware operates as a hierarchical state machine with well-defined transitions:

### 3.1.1 State Definitions

| State | Description |
| --- | --- |
| POWER_ON_RESET | Hardware initialization, clock config, peripheral setup. Duration: <100ms |
| IDLE | Waiting for valid input signal. All outputs tri-stated. Beep sequence available. |
| ARMED | Valid signal received, throttle = 0. Ready for motor start. PWM outputs ready but OFF. |
| ALIGN | Rotor alignment phase. Fixed commutation state applied. Duration: configurable 50-500ms. |
| OPEN_LOOP_RAMP | Forced commutation at increasing frequency. No BEMF feedback. Duration: until target RPM. |
| CLOSED_LOOP | Sensorless operation using BEMF zero-crossing. Normal running state. |
| BRAKING | Active regenerative braking (if enabled). Controlled deceleration. |
| FAULT | Fault condition detected. All outputs disabled. Requires throttle=0 to clear. |

### 3.1.2 State Transition Table

| From | Event/Condition | To | Action |
| --- | --- | --- | --- |
| POWER_ON_RESET | Init complete | IDLE | Start signal detect |
| IDLE | Valid signal, throttle=0 | ARMED | Arm beep |
| ARMED | Throttle > MIN_START | ALIGN | Begin alignment |
| ARMED | Signal loss (>100ms) | IDLE | Disarm |
| ALIGN | Align time complete | OPEN_LOOP_RAMP | Start ramp |
| OPEN_LOOP_RAMP | BEMF detected, stable | CLOSED_LOOP | Switch to sensorless |
| OPEN_LOOP_RAMP | Ramp timeout, no BEMF | FAULT | Startup failure |
| CLOSED_LOOP | Throttle = 0 | ARMED | Motor stop |
| CLOSED_LOOP | Desync detected | OPEN_LOOP_RAMP | Resync attempt |
| CLOSED_LOOP | Signal loss (>100ms) | BRAKING | Controlled stop |
| ANY | Overcurrent fault | FAULT | Immediate disable |
| FAULT | Throttle=0 for >500ms | ARMED | Clear fault |

## 3.2 Commutation Timing and Windows

### 3.2.1 Six-Step Commutation Table

| Step | A | B | C | BEMF Phase | ZC Direction | Comp Pol |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | HIGH | LOW | Float | C | Rising | + |
| 2 | HIGH | Float | LOW | B | Falling | - |
| 3 | Float | HIGH | LOW | A | Rising | + |
| 4 | LOW | HIGH | Float | C | Falling | - |
| 5 | LOW | Float | HIGH | B | Rising | + |
| 6 | Float | LOW | HIGH | A | Falling | - |

### 3.2.2 Critical Timing Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Blanking Time (post-commutation) | 5-20% of step | Ignore comparator during switching noise |
| Demagnetization Time | 10-35% of step | Wait for floating phase current decay |
| BEMF Sampling Window | Center of PWM ON | Synchronized with ADC trigger |
| ZC-to-Commutation Delay | 30° electrical | Half of 60° step, adjustable for timing advance |
| Timing Advance Range | 0-30° | Configurable, typically 15° for drone motors |
| Min Commutation Period | 10 µs | ~166,000 eRPM max (hardware limit) |
| Max Commutation Period | 65 ms | Below this, transition to open-loop/stop |

### 3.2.3 BEMF Sampling Strategy

BEMF zero-crossing detection uses synchronized ADC sampling during PWM ON time:

- PWM center triggers ADC conversion of floating phase

- Compare sampled voltage against virtual neutral (Vbus/2)

- Apply majority filter: 3 consecutive same-polarity samples = valid

- Detect transition from majority-LOW to majority-HIGH (or vice versa)

- Calculate next commutation time: T_next = T_zc + (30° - advance) delay

### 3.2.4 Desync Detection Algorithm

Desynchronization is detected when commutation timing becomes inconsistent:

- If commutation period variance > 25% of average over 6 steps → desync warningVariance Check:

- If no zero-crossing detected within 150% of expected time → force commutationMissing ZC:

- If 3+ consecutive forced commutations → desync confirmedConsecutive Misses:

- Reduce duty cycle 50%, attempt resync. If 3 resync attempts fail → FAULT stateRecovery:

- Track forced_commutations / total_commutations ratio for EDT stress levelStress Metric:

## 3.3 Advanced Motor Control Algorithms

Project Garuda implements state-of-the-art motor control techniques that leverage the dsPIC33AK's hardware FPU and CLC modules to achieve performance exceeding traditional 6-step implementations.

### 3.3.1 Flux Linkage Integration (Low-Speed Enhancement)

Traditional zero-crossing detection fails below ~10% throttle due to insufficient BEMF amplitude. Flux linkage integration enables robust sensorless operation from near-zero speed.

Principle:

- Instead of detecting instantaneous ZC events, integrate the flux linkage function over time

- Commutation triggered when integrated flux reaches threshold (speed-independent)

- Acts as massive noise filter - immune to PWM switching transients

Implementation:

- Sample BEMF at PWM center (existing ADC trigger)

- FPU accumulates: Ψ += (V_bemf - V_neutral) × dt

- Commutation when |Ψ| exceeds threshold (auto-scaled by speed estimate)

- Reset integrator after each commutation

Performance Target:

- Sensorless operation down to 2% throttle (vs 10% for pure ZC)

- Enables smooth cinematography drone operation

### 3.3.2 Phase Voltage Integration (Area-Based Commutation)

Hybrid approach combining ZC detection with area-based validation for noise immunity:

| Method | Description |
| --- | --- |
| Primary: ZC Detection | Hardware comparator detects zero-crossing (fast response) |
| Validation: Area Integration | ADC integrates BEMF area; reject ZC if area < threshold (noise filter) |
| Fallback: Area-Only Mode | At low speeds, use pure integration when ZC unreliable |

This dual-mode approach provides the speed of hardware ZC detection with the noise immunity of integration.

### 3.3.3 Ternary Hysteresis Control (Torque Ripple Reduction)

The primary weakness of 6-step control is the torque 'kick' every 60° electrical. Ternary hysteresis control reduces this ripple by 40-60% without FOC computational overhead.

Three-State Logic:

| State | Condition | Action |
| --- | --- | --- |
| Active Vector | torque_error > +hysteresis | Normal commutation (drive current into motor) |
| Zero Vector | ¦torque_error¦ < hysteresis | All high-side ON or all low-side ON (coast) |
| Reverse Vector | torque_error < -hysteresis | Regenerative braking (absorb energy) |

Implementation:

- Hysteresis band: Configurable 5-15% of rated torque (default 10%)

- CLC implements 3-state comparator with programmable thresholds

- Zero vector insertion during torque error dead-band smooths transitions

- Result: Approaches FOC smoothness at fraction of CPU cost

### 3.3.4 Adaptive Lead Angle Compensation

Fixed timing advance (current spec: 0-30°) is suboptimal across the operating range. Adaptive lead angle dynamically adjusts based on operating conditions:

| Factor | Effect | Compensation |
| --- | --- | --- |
| Motor Speed (eRPM) | Higher speed → more inductance lag | Increase advance: +1° per 10k eRPM |
| Phase Current | Higher current → more lag | Increase advance: +0.5° per 10A |
| Temperature | Higher temp → less inductance | Decrease advance: -0.2° per 10°C above 25°C |

Algorithm:

advance = base_advance + (eRPM/10000) + (I_phase/20) - ((T-25)/50)

- Range: 0-45° (extended from fixed 0-30°)

- Update rate: Every commutation cycle

- FPU computes in <500ns

### 3.3.5 Mixed Decay Mode (Acoustic Optimization)

PWM decay mode affects both current ripple and acoustic noise. Adaptive mixed decay optimizes for operating conditions:

| Mode | Characteristics | Best For |
| --- | --- | --- |
| Slow Decay | Current recirculates through motor; low acoustic noise | Low speed, quiet operation |
| Fast Decay | Current returns to supply; lower ripple, higher noise | High speed, max efficiency |
| Mixed Decay | Dynamically blend within single PWM cycle | Adaptive (default) |

Adaptive Strategy:

- Below 20% throttle: Slow decay (prioritize quiet operation)

- Above 80% throttle: Fast decay (prioritize efficiency)

- 20-80% throttle: Mixed decay with ratio based on throttle rate of change

- Implementation: PWM dead-time and override registers updated per cycle

### 3.3.6 CLC-Accelerated Commutation State Machine

The 6-step commutation truth table is implemented entirely in hardware using CLC modules, achieving sub-10ns switching latency:

Hardware Implementation:

- Input: 3-bit sector register (updated by CPU at each commutation)

- Output: 6 PWM override signals directly to gate drivers

- Logic: Combinatorial truth table in CLC (no clock delays)

- Latency: <10ns from sector update to gate drive change

CLC Configuration:

| Sector | AH | AL | BH | BL | CH / CL |
| --- | --- | --- | --- | --- | --- |
| 0 (0°) | PWM | OFF | OFF | ON | Float |
| 1 (60°) | PWM | OFF | Float | Float | OFF / ON |
| 2 (120°) | Float | Float | PWM | OFF | OFF / ON |
| 3 (180°) | OFF | ON | PWM | OFF | Float |
| 4 (240°) | OFF | ON | Float | Float | PWM / OFF |
| 5 (300°) | Float | Float | OFF | ON | PWM / OFF |

PWM = Modulated by duty cycle; ON = Always high; OFF = Always low; Float = High-impedance (BEMF sensing)

Advantage over Software:

- Eliminates ISR latency jitter (critical above 100k eRPM)

- CPU only updates 3-bit sector register; CLC handles all 6 outputs

- Enables consistent timing across varying CPU load

### 3.3.7 Algorithm Phase-In Schedule

| Phase | Algorithm | Dependency |
| --- | --- | --- |
| Phase 2 | CLC State Machine + Basic ZC | Core sensorless operation |
| Phase 2 | Phase Voltage Integration (hybrid) | Enhanced noise immunity |
| Phase 4 | Adaptive Lead Angle | Requires current sensing |
| Phase 4 | Mixed Decay Mode | Acoustic optimization |
| Phase 5 | Flux Linkage Integration | Low-speed enhancement |
| Phase 5 | Ternary Hysteresis Control | Torque ripple reduction |

# 4. Configuration and EEPROM System

## 4.1 EEPROM Binary Layout

Configuration stored in little-endian format. Total size: 64 bytes (expandable to 128).

| Offset | Size | Type | Field Name | Default |
| --- | --- | --- | --- | --- |
| 0x00 | 2 | uint16 | MAGIC (0x4D50 = 'MP') | 0x4D50 |
| 0x02 | 1 | uint8 | SCHEMA_VERSION | 0x01 |
| 0x03 | 1 | uint8 | HARDWARE_ID | Target-specific |
| 0x04 | 2 | uint16 | FIRMWARE_VERSION | 0x0100 |
| 0x06 | 1 | uint8 | MOTOR_DIRECTION | 0x00 (normal) |
| 0x07 | 1 | uint8 | MOTOR_POLES | 14 |
| 0x08 | 2 | uint16 | MOTOR_KV | 0 (auto) |
| 0x0A | 1 | uint8 | TIMING_ADVANCE (0-30) | 15 |
| 0x0B | 1 | uint8 | PWM_FREQ_KHZ | 24 |
| 0x0C | 1 | uint8 | STARTUP_POWER (0-2) | 1 (medium) |
| 0x0D | 1 | uint8 | FLAGS_1 (bitfield) | 0x03 |
| 0x0E | 1 | uint8 | FLAGS_2 (bitfield) | 0x00 |
| 0x0F | 1 | uint8 | BEEP_VOLUME (0-3) | 2 (medium) |
| 0x10 | 1 | uint8 | BRAKE_STOP_LEVEL (%) | 0 |
| 0x11 | 1 | uint8 | BRAKE_RUN_LEVEL (%) | 100 |
| 0x12-0x3D | 44 | - | Reserved (zero-filled) | 0x00 |
| 0x3E | 2 | uint16 | CRC-16-CCITT | Calculated |

### 4.1.1 FLAGS_1 Bitfield

- Bit 0: COMP_PWM (Complementary PWM) - default 1

- Bit 1: VAR_PWM (Variable PWM frequency) - default 1

- Bit 2: BIDIR_MODE (Bidirectional/3D mode) - default 0

- Bit 3: STUCK_PROTECT (Stuck rotor protection) - default 0

- Bit 4: BRAKE_ON_STOP - default 0

- Bit 5: STALL_PROTECT - default 0

- Bit 6: SINE_STARTUP - default 0

- Bit 7: Reserved

### 4.1.2 Versioning and Compatibility

- SCHEMA_VERSION increment = breaking change (new fields, removed fields)

- Forward compatibility: Unknown fields preserved, reserved areas zeroed

- CRC-16-CCITT (poly 0x1021, init 0xFFFF) over bytes 0x00-0x3D

- On CRC failure: Reset to factory defaults, log error

### 4.1.3 Flash Wear Management

- dsPIC33AK uses emulated EEPROM in Flash (DEE library)

- Minimum 100,000 erase cycles per page

- Wear leveling: 4KB reserved area with 4x redundancy

- Write coalescing: Buffer changes, commit on SAVE_SETTINGS command

# 5. Bootloader and Firmware Update

## 5.1 Bootloader Architecture

| Component | Specification |
| --- | --- |
| Bootloader Size | 4 KB (0x0000-0x0FFF) |
| Application Start | 0x1000 |
| Application Max Size | 120 KB |
| Config/EEPROM Area | Last 4 KB of flash |

## 5.2 Update Transports

### 5.2.1 1-Wire Passthrough (Primary)

- Uses DShot signal wire for bidirectional communication

- Compatible with Betaflight/ArduPilot passthrough protocol

- Baud rate: 19200 (configurable)

- Entry: Special DShot command sequence (CMD 255 x 10)

### 5.2.2 UART (Secondary)

- Dedicated UART pads (if available on hardware)

- Baud rate: 115200

- Entry: Hold signal LOW during power-up

## 5.3 Firmware Image Format

- Header: 32 bytes (magic, version, size, target ID, CRC-32)

- Payload: Raw binary, encrypted if security enabled

- Footer: CRC-32 of entire image

## 5.4 Safety and Recovery

- Active and backup slots; fallback on boot failureDual-image (A/B):

- CRC-32 check before boot; reject corrupted imagesValidation:

- 500ms timeout; reset to bootloader on hangWatchdog:

- Bootloader always accessible via UART; cannot be overwrittenAnti-brick:

- On 3 consecutive boot failures, revert to previous imageRollback:

## 5.5 Optional Security Features

- Ed25519 signature verification (optional)Signed Images:

- AES-128-CTR encryption (optional)Encrypted Images:

- Lock bootloader, require signed images onlySecure Boot:

# 6. Real-Time Architecture

## 6.1 Interrupt Priority Scheme

| Priority | Source | Handler / Notes |
| --- | --- | --- |
| 7 (highest) | PWM Fault (comparator) | Immediate output disable. <1µs response. Non-maskable. |
| 6 | Comparator ZC | Zero-crossing event. Capture timestamp, set commutation timer. |
| 5 | Commutation Timer | Execute commutation. Update PWM override registers. |
| 4 | ADC Complete | BEMF sample processing. Majority filter update. |
| 3 | DShot Input Capture | Capture pulse widths to buffer. DMA handles transfer. |
| 2 | DShot Frame Complete | Decode frame, update throttle setpoint, queue telemetry. |
| 1 | 1ms System Tick | State machine, timeout checks, telemetry scheduling. |
| 0 (lowest) | Background | EEPROM writes, diagnostics, idle loop. |

## 6.2 Timing Budget (per commutation step)

| Operation | Max Time | Notes |
| --- | --- | --- |
| PWM Fault ISR | 200 ns | Hardware-assisted disable |
| Comparator ZC ISR | 500 ns | Timestamp capture only |
| Commutation ISR | 1 µs | PWM override update |
| ADC + Majority Filter | 2 µs | 3 samples @ 40 MSPS |
| DShot Decode | 5 µs | CRC check, command parse |
| PI Control Loop | 2 µs | FPU-accelerated |
| Total Critical Path | <10 µs | Supports >100k eRPM |

## 6.3 Timing Source Hierarchy

- Primary: PWM timebase (200 MHz / prescaler) - all motor timing derived from this

- Secondary: Comparator edge timestamps (captured by SCCP relative to PWM timebase)

- Tertiary: System tick (1 ms) for state machine and timeout management

# 7. Configuration Interface Protocol

## 7.1 Protocol Selection Strategy

The configurator protocol determines end-user tooling compatibility. After analysis, MicroPilot uses a custom protocol (MPSP) with MSP subset for FC passthrough.

| Option | Pros | Cons | IP Status |
| --- | --- | --- | --- |
| Custom (MPSP) | Full control, WebUSB ready, no IP risk | Requires custom PC tool | ✅ Clean |
| KISS Emulation | Works with existing tools | Limited features, IP grey area | ⚠️ Grey |
| MSP Subset | Well-documented, FC passthrough | Overkill for standalone ESC | ✅ Clean |

Decision: Option A (Garuda Serial Protocol - GSP) as primary + Option C (MSP subset) for FC passthrough compatibility.

## 7.2 Garuda Serial Protocol (GSP)

### 7.2.1 Transport Layer

| Parameter | Value |
| --- | --- |
| Physical | UART (1-wire half-duplex via DShot pad) or dedicated TX/RX |
| Baud Rate | 115200 default (negotiable to 921600) |
| Frame Format | 8N1 |
| Flow Control | None (packet-based with ACK/NAK) |

### 7.2.2 Packet Structure

All packets follow this structure:

| START | LENGTH | COMMAND | PAYLOAD | CRC |
| --- | --- | --- | --- | --- |
| 1 byte | 1 byte | 1 byte | 0-252 bytes | 2 bytes |
| 0x24 ('$') | N+3 | CMD | DATA | CRC16-CCITT |

CRC: CRC16-CCITT (poly 0x1021, init 0xFFFF) over LENGTH+COMMAND+PAYLOAD bytes.

### 7.2.3 Command Set

| CMD | Name | Direction | Description |
| --- | --- | --- | --- |
| 0x01 | GET_INFO | Req→ESC | Request device info (FW version, HW ID, capabilities) |
| 0x81 | INFO_RESP | ESC→Req | 32-byte response with device information |
| 0x02 | GET_CONFIG | Req→ESC | Request full configuration blob |
| 0x82 | CONFIG_RESP | ESC→Req | 64-byte current configuration |
| 0x03 | SET_CONFIG | Req→ESC | Write new configuration (64 bytes) |
| 0x83 | SET_RESP | ESC→Req | Status: 0=OK, 1=CRC error, 2=Invalid |
| 0x04 | SAVE_CONFIG | Req→ESC | Commit configuration to EEPROM |
| 0x05 | RESET_DEFAULT | Req→ESC | Factory reset to defaults |
| 0x06 | ENTER_BOOTLOADER | Req→ESC | Reboot into bootloader mode |
| 0x07 | GET_TELEMETRY | Req→ESC | Request live telemetry (RPM, V, I, T) |
| 0x08 | MOTOR_TEST | Req→ESC | Spin motor at specified duty (0-2047) |
| 0x09 | BEEP | Req→ESC | Play beep pattern (1-5) |
| 0xFF | NAK | ESC→Req | Error response with error code |

### 7.2.4 INFO_RESP Payload (32 bytes)

| Offset | Size | Field | Example/Notes |
| --- | --- | --- | --- |
| 0x00 | 4 | Magic ('GRDA') | 0x47524441 |
| 0x04 | 2 | Protocol Version | 0x0100 (v1.0) |
| 0x06 | 2 | Firmware Version | 0x0103 (v1.3) |
| 0x08 | 4 | Hardware ID | Board-specific |
| 0x0C | 4 | Capabilities Flags | Bitfield (see below) |
| 0x10 | 16 | Device Name | "GARUDA-33AK" (null-padded) |

Capabilities Flags:

- Bit 0: Bidirectional DShot supported

- Bit 1: EDT telemetry supported

- Bit 2: 3D mode (bidirectional motor) supported

- Bit 3: Sinusoidal startup supported

- Bit 4: DroneCAN supported

- Bit 5: Current sensing available

- Bit 6: Temperature sensing available

### 7.2.5 TELEMETRY_RESP Payload (16 bytes)

| Offset | Size | Field | Units |
| --- | --- | --- | --- |
| 0x00 | 4 | eRPM | Raw eRPM value |
| 0x04 | 2 | Voltage | millivolts (mV) |
| 0x06 | 2 | Current | milliamps (mA), signed |
| 0x08 | 2 | Temperature | 0.1°C units |
| 0x0A | 1 | Duty Cycle | 0-255 (0-100%) |
| 0x0B | 1 | State | Current state machine state |
| 0x0C | 2 | Error Flags | Bitfield |
| 0x0E | 2 | Reserved | 0x0000 |

## 7.3 FC Passthrough (MSP Subset)

For configuration through flight controllers (Betaflight passthrough), the ESC supports MSP commands in addition to native GSP:

### 7.3.1 MSP Frame Format

Standard MSP v1 framing: "$M<" (request) or "$M>" (response) + LENGTH + CMD + PAYLOAD + XOR checksum.

### 7.3.2 Supported MSP Commands

| MSP_CMD | Name | Description |
| --- | --- | --- |
| 126 (0x7E) | MSP_ESC_INFO | Get ESC information |
| 127 (0x7F) | MSP_ESC_CONFIG | Get/Set ESC configuration |
| 128 (0x80) | MSP_ESC_FLASH | Write configuration to flash |

Note: These are custom MSP command IDs in the user-defined range (≥126).

## 7.4 Bootloader Protocol

### 7.4.1 Entry Methods

- DShot Command Sequence: Send DShot command 255 (0xFF) 10 times within 500ms

- UART: Hold signal LOW during power-up, send 0x00 within 100ms

- Software: ENTER_BOOTLOADER command via GSP

### 7.4.2 Bootloader Commands

| CMD | Name | Description |
| --- | --- | --- |
| 0x00 | BL_PING | Verify bootloader active, returns ACK |
| 0x01 | BL_GET_INFO | Get bootloader version, flash size, page size |
| 0x02 | BL_ERASE | Erase application area (preserves bootloader + config) |
| 0x03 | BL_WRITE | Write 256-byte page: CMD + ADDR(4) + DATA(256) + CRC(2) |
| 0x04 | BL_READ | Read 256-byte page for verification |
| 0x05 | BL_VERIFY | Calculate CRC of written image, compare with expected |
| 0x06 | BL_BOOT | Exit bootloader, start application |

### 7.4.3 Firmware Update Sequence

- BL_PING → Verify bootloader response

- BL_GET_INFO → Confirm flash size matches image

- BL_ERASE → Erase application area

- BL_WRITE × N → Write firmware pages (256 bytes each)

- BL_VERIFY → Calculate and compare CRC

- BL_BOOT → Start application

## 7.5 PC Configuration Tool

### 7.5.1 Recommended Implementation

Technology Stack:

- Cross-platform: Electron + React (or Tauri for smaller footprint)

- Serial: WebSerial API (browser-based) or node-serialport (desktop)

- Alternative: WebUSB for direct USB CDC connection

Core Features:

- Auto-detect ESC on serial ports

- Read/display current configuration with validation

- Live telemetry dashboard (RPM, voltage, current, temperature graphs)

- Motor test with safety timeout (max 5 seconds)

- Firmware update via bootloader with progress indicator

### 7.5.2 Safety Interlocks

- Requires explicit user confirmation + auto-stops after 5s or on disconnectMotor test:

- Changes require explicit SAVE_CONFIG to persistConfiguration:

- Requires battery disconnection confirmationFirmware update:

- Immediate motor stop command on serial disconnectConnection loss:

## 7.6 Dead Time Auto-Calibration

Addressing the hardware-specific dead time concern from review:

### 7.6.1 Calibration Procedure

At first power-up or on explicit calibration command:

- Apply minimum dead time (50ns), duty cycle 1%

- Gradually increase duty to 5%, monitor DC bus current via ADC

- If current spike > 2× expected → shoot-through detected → increase dead time by 50ns

- Repeat until no shoot-through detected

- Set optimal = detected minimum + 50ns safety margin

- Store calibrated value to EEPROM, mark as calibrated

### 7.6.2 Safe Defaults (Pre-Calibration)

- 300ns (safe for most MOSFETs)Default dead time:

- 90% until calibration completeMax duty limit:

- Warning in EDT telemetry indicating uncalibratedStatus flag:

# Appendix A: Clean-Room Compliance

## A.1 Purpose and Scope

This appendix defines the clean-room development practices ensuring Project Garuda ESC firmware is developed with complete IP independence and no licensing obligations from third-party ESC codebases.

## A.2 Source Classification

### A.2.1 APPROVED Sources

Implementation SHALL be based only on these source classes:

| Source Class | Examples |
| --- | --- |
| Microchip Documentation | dsPIC33AK datasheets, FRM, application notes (AN1160, etc.) |
| Microchip Code | Motor Control Library (permissively licensed), MCC-generated code |
| Public Protocol Specs | See A.3 - independently verified protocol behavior |
| Academic/Textbook | Motor control theory, DSP algorithms (with citation) |
| Empirical Measurement | Logic analyzer captures, oscilloscope traces of protocol behavior |
| Original Development | Code written by Project Garuda team without reference to prohibited sources |

### A.2.2 PROHIBITED Sources

Implementation SHALL NOT reference or derive from:

- BLHeli, BLHeli_S, BLHeli_32 source code or binaries

- AM32 source code (GPL-3.0 licensed)

- Bluejay source code (GPL-3.0 licensed)

- KISS ESC firmware (proprietary)

- Disassembled or decompiled ESC firmware

- Leaked or NDA-protected documentation

## A.3 Protocol Reference Sources

Protocol specifications are based on publicly available documentation with licenses noted:

| Reference | URL | License |
| --- | --- | --- |
| Betaflight DShot Docs | betaflight.com/docs/development/Dshot | CC-BY-SA 4.0 |
| DShot Missing Handbook | brushlesswhoop.com/dshot-and-bidirectional-dshot | Blog (factual) |
| EDT Specification | github.com/bird-sanctuary/extended-dshot-telemetry | GPL-3.0 (spec) |
| ArduPilot DShot Docs | ardupilot.org/copter/docs/common-dshot-escs | CC-BY-SA 3.0 |

Note on EDT Specification: The bird-sanctuary/extended-dshot-telemetry repository is GPL-3.0 licensed. Our specification uses it as ONE corroborating reference for protocol facts, combined with:

- Independent logic analyzer captures of EDT-enabled ESCs

- Betaflight flight controller source (GPL-3.0, FC-side only)

- Multiple community forum discussions documenting observed behavior

Protocol facts (frame structure, timing) are not copyrightable. Implementation is original.

## A.4 Role Separation

- May review public protocol documentation to define requirementsSpecification Author:

- Works from specification only; SHALL NOT view prohibited source codeImplementer:

- Tests against protocol conformance; documents deviationsVerifier:

## A.5 Requirement Traceability

Each requirement in this specification SHALL be tagged with its source:

- [MCHP-ANxxxx]: Microchip Application Note

- [MCHP-DS]: Microchip Datasheet/FRM

- [PROTO-DSHOT]: DShot protocol (public specification)

- [PROTO-EDT]: EDT protocol (observed behavior + public spec)

- [EMPIRICAL]: Measured/captured behavior

- [ORIGINAL]: Original design decision

# Appendix B: Verification Plan

## B.1 Protocol Conformance Tests

| Test | Method | Pass Criteria |
| --- | --- | --- |
| DShot150/300/600/1200 decode | Logic analyzer + reference FC | 100% frame decode |
| CRC validation | Inject bad CRC frames | All rejected |
| Bidir mode detection | Switch FC bidir on/off | Auto-detect <10ms |
| eRPM telemetry accuracy | Optical tachometer reference | ±2% error |
| EDT frame decode (FC side) | Betaflight blackbox | All frames valid |
| Telemetry BER | 100k frames, count errors | <0.1% error rate |

## B.2 Motor Control Tests

| Test | Method | Pass Criteria |
| --- | --- | --- |
| Startup reliability | 100 cold starts, various motors | >98% success |
| Speed range | Dyno test, throttle sweep | 1000-50000 eRPM |
| Throttle linearity | Dyno test, RPM vs throttle | R² > 0.99 |
| Desync recovery | Prop strike simulation | Recovery <100ms |
| Direction change (3D) | Throttle reversal test | Clean transition |

## B.3 Protection Tests

| Test | Method | Pass Criteria |
| --- | --- | --- |
| Overcurrent protection | Stall motor, measure trip | Trip <1ms |
| Overvoltage protection | Inject 30V spike | No damage |
| Undervoltage cutoff | Ramp down supply voltage | Clean shutdown |
| Thermal protection | Heat sink removal test | Derating active |
| Signal loss failsafe | Disconnect FC signal | Stop <500ms |

## B.4 Environmental Tests

- Temperature range: -20°C to +85°C operation

- Vibration: Per MIL-STD-810G, Method 514.7

- EMC: Per EN 55032 Class B emissions

# Appendix C: Development Hardware

## C.1 Recommended Development Boards

| Board | Part Number | Purpose |
| --- | --- | --- |
| dsPIC33A Curiosity Board | EV74H48A | General development |
| dsPIC33AK128MC106 Motor Control DIM | EV68M17A | Motor control focus |
| MCLV-2 Development Board | DM330021-2 | Low-voltage motor control |
| Hurst 24V BLDC Motor | AC300020 | Reference motor |

## C.2 Test Equipment

- Logic analyzer: Saleae Logic Pro 16 or equivalent (>100 MHz sampling)

- Oscilloscope: 4-channel, >200 MHz bandwidth

- Power supply: 0-30V, 10A programmable

- Motor dyno: Thrust stand with RPM, current, thrust measurement

- Thermal camera: For temperature verification

## C.3 Software Tools

- MPLAB X IDE v6.x with XC-DSC compiler

- MPLAB Code Configurator (MCC) for peripheral setup

- MPLAB Data Visualizer for real-time debugging

- Betaflight Configurator for FC-side testing

- BLHeli Suite (read-only, for compatibility verification)

# Appendix D: Development Roadmap

## D.1 Phase Overview

| Phase | Duration | Deliverables |
| --- | --- | --- |
| 1: Foundation | Weeks 1-4 | Clock config, PWM init, ADC init, basic commutation, motor spins open-loop |
| 2: Sensorless | Weeks 5-8 | BEMF detection, majority filter, startup sequence, closed-loop control, PI loop |
| 3: Protocols | Weeks 9-12 | DShot receive, commands, bidirectional DShot, EDT telemetry, full compatibility |
| 4: Production | Weeks 13-16 | Protection features, EEPROM config, bootloader, motor tuning, production-ready |
| 5: Extended | Weeks 17-20 | DroneCAN (optional), sinusoidal startup, advanced algorithms, safety documentation |

## D.2 Success Criteria

- Phase 1: Motor spins with manual commutation sequence

- Phase 2: Reliable sensorless startup on 3+ different motor types

- Phase 3: Full DShot600 + EDT compatibility with Betaflight

- Phase 4: Pass all verification tests in Appendix B

- Phase 5: Complete reference design package ready for production

## D.3 Risk Mitigations

- Test with diverse motor types from Phase 2Motor compatibility:

- Extensive FC compatibility testingProtocol edge cases:

- Early thermal characterization with dynoThermal issues:

- Clean-room audit at Phase 2 milestoneIP concerns: